# RIP Routing Configuration Using 3 Routers

Routing Information Protocol (RIP) is an active routing protocol that operates hop count as a routing metric to find the most suitable route between the source and the destination network. It is a distance-vector routing protocol that has an AD value of 120 and works on the Network layer of the OSI model.

### Steps to Configure and Verify Three Router Connections in Cisco Packet Tracer using RIP Routing:

Step 1: First, open the Cisco packet tracer desktop and select the devices given below:

| S.NO | Device | Model Name | Qty. |
| --- | --- | --- | --- |
| 1. | PC | PC | 6 |
| 2. | Switch | PT-Switch | 3 |
| 3. | Router | PT-router | 3 |

IP Addressing Table:

| S.NO | Device | IPv4 Address | Subnet mask | Default Gateway |
| --- | --- | --- | --- | --- |
| 1. | PC0 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| 2. | PC1 | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 |
| 3. | PC2 | 192.168.20.2 | 255.255.255.0 | 192.168.20.1 |
| 4. | PC3 | 192.168.20.3 | 255.255.255.0 | 192.168.20.1 |
| 5. | PC4 | 192.168.30.2 | 255.255.255.0 | 192.168.30.1 |
| 6. | PC5 | 192.168.30.3 | 255.255.255.0 | 192.168.30.1 |

- Then, create a network topology as shown below the image.

- Use an Automatic connecting cable to connect the devices with others.

Step 2: Configure the PCs (hosts) with IPv4 address and Subnet Mask according to the IP addressing table given above.

- To assign an IP address in PC0, click on PC0.

- Then, go to desktop and then IP configuration and there you will IPv4 configuration.

- Fill IPv4 address and subnet mask.

- Assigning an IP address using the ipconfig command, or we can also assign an IP address with the help of a command.

- Go to the command terminal of the PC.

- Then, type iPConfig <IPv4 address><subnet mask><default gateway>(if needed)

Example: iPConfig 192.168.10.2  255.255.255.0 192.168.10.1

- Repeat the same procedure with other PCs to configure them thoroughly.

Step 3: Configure router with IP address and Subnet mask.

IP Addressing Table Router:

| S.NO | Device | Interface | IPv4 Address | Subnet mask |
| --- | --- | --- | --- | --- |
| 1. | router0 | FastEthernet0/0 | 192.168.10.1 | 255.255.255.0 |
|  |  | Serial2/0 | 10.0.0.1 | 255.0.0.0 |
| 2. | router1 | FastEthernet0/0 | 192.168.20.1 | 255.255.255.0 |
|  |  | Serial2/0 | 10.0.0.2 | 255.0.0.0 |
|  |  | Serial3/0 | 11.0.0.1 | 255.0.0.0 |
| 3. | router2 | FastEthernet0/0 | 192.168.30.1 | 255.255.255.0 |
|  |  | Serial2/0 | 11.0.0.2 | 255.0.0.0 |

- To assign an IP address in router0, click on router0.

- Then, go to config and then Interfaces.

- Make sure to turn on the ports.

- Then, configure the IP address in FastEthernet and serial ports according to IP addressing Table.

- Fill IPv4 address and subnet mask.

- Repeat the same procedure with other routers to configure them thoroughly.

Step 4: After configuring all of the devices we need to assign the routes to the routers.

To assign RIP routes to the particular router:

- First, click on router0 then Go to CLI.

- Then type the commands and IP information given below.

CLI command : network <network id>

RIP Routes for Router0 are given below:

Router(config)#network 192.168.10.0

Router(config)#network 10.0.0.0

RIP Routes for Router1 are given below:

Router(config)#network 192.168.20.0

Router(config)#network 10.0.0.0

Router(config)#network 11.0.0.0

RIP Routes for Router2 are given below:

Router(config)#network 192.168.30.0

Router(config)#network 11.0.0.0

Step 5: Verifying the network by pinging the IP address of any PC.

- We will use the ping command to do so.

- First, click on PC0 then Go to the command prompt.

- Then type ping <IP address of targeted node>.

- As we can see in the below image we are getting replies which means the connection is working properly.

Example : ping 192.168.20.2

- A simulation of the experiment is given below we are sending PDU from  PC0 to PC2 and PC3 to PC5: